NAVA Guideline Summary (updated December 16, 2025)

Considerations:

  - Struggling to wean utilizing traditional ventilator approaches

  - Babies with heterogenous lung disease may benefit from more intrinsic breathing control

  - Babies with established BPD (bronchopulmonary dysplasia)

  - Extubation modality for babies who would benefit from increased ventilatory support than CPAP

  - Babies with frequent apneic spells

Neurally Adjusted Ventilatory Assist (NAVA) provides proportional ventilation by synchronizing with the diaphragm’s electrical activity (Edi signal) detected via a specialized NG/OG catheter.

Contraindications:

- Absent respiratory effort.

- Esophageal atresia,

- Unrepaired CDH,

- Phrenic nerve injury,

- Congenital myopathy.

- MRI (remove catheter before scan).

Adjust NAVA level only when Edi peaks/mins remain outside the target range for at least one hour, using trend data to avoid frequent changes that may destabilize the infant.

Obtain x-ray every time NAVA catheter is replaced.

Suggested Settings and Management Strategies:

|  |  | Initial Settings | Range (Suggested Minimum & Maximum) |  |  | Adjust by |
| --- | --- | --- | --- | --- | --- | --- |
| NAVA Level (optimize NAVA level according to Edi Max) |  | 1-2 cm H2O/µV | 0.5cm H2O/µV-4-6 cm H2O/µV Do not use NAVA <1 for intubated patients |  |  | 0.1-0.2 cm H2O/µV |
| PEEP, cm H2O |  | 6-8 cm H2O | Max 14 cm H2O -Consideration to be given based on interface -Same or one higher than ventilator settings -Adjust based on gases and x-ray |  |  | 1cm H2O |
| Consider increasing PEEP when transitioning from invasive to NIV NAVA to maintain adequate MAP |  |  |  |  |  |  |
| Edi Goal: Minimum: 2-4 µV Target: 10-15µV) |  |  | - For Edi Max >15µV, increase NAVA level. - If Edi Max <10µV decrease NAVA level - If Edi min >5µV increase PEEP - If Edi Min too low Decrease PEEP |  |  |  |
| Backup Settings |  |  |  |  |  |  |
|  | Initial Settings |  |  | Range (Suggested Minimum & Maximum) | Adjust by |  |
| Backup pressure control above PEEP | 10-15 cm H2O |  |  | Max 30-35cm H2O (or per physician discretion) | 1-2cm H2O |  |
| Rate | 30-40/min |  |  | Max of 60/min | As needed |  |
| Inspiratory Time | 0.3-0.5 seconds |  |  | 0.3-0.7 seconds | 0.05-0.1 seconds |  |
| Apnea Time | 2-3 seconds |  |  | 2-15 seconds | 1-2 seconds |  |
| Trigger Sensitivity | 1-2 |  |  |  | - |  |
| High-pressure alarm | 40-50cm H2O (or higher as needed) |  |  |  | - |  |

Key Considerations

- CXR required after placement to confirm catheter position.

- If frequent backup ventilation occurs, consider:

  - Increasing NAVA Level.

  - Reducing apnea time.

  - Checking Edi catheter placement.

- Avoid RAM cannula for NIV-NAVA due to high resistance.

Weaning Strategy

- Decrease NAVA Level if pCO₂ <40 (apnea) or pCO₂ <55 (BPD).

- Increase NAVA Level if pCO₂ >65-70 or increased WOB.

- Transition to CPAP when stable on low NAVA settings.